Deactivating and Activating the

Belt-Minder Feature

WARNING: While the system

allows you to deactivate it, this system

is designed to improve your chances of

being safely belted and surviving an

accident. We recommend you leave the

system activated for yourself and others

who may use the vehicle.

Note: The driver and front passenger

warnings switch on and off independently.

When you perform this procedure for one

seating position, do not buckle the other

position as this will terminate the process.

Note: If you are using MyKey, you cannot

disable the Belt-Minder. Also, if you have

previously disabled the Belt-Minder, it will

be re-enabled during the use of MyKey.

Read Steps 1 - 4 thoroughly before

proceeding with the programming

procedure.

Before following the procedure, make sure

that:

• You set the parking brake.

• You put the transmission in park (P)

(automatic transmission) or neutral

(N) (manual transmission).

• You switch the ignition off.

• You unbuckle the driver and front

passenger seatbelts.

1. Switch the ignition on. Do not start the

vehicle.

2. Wait until the seatbelt warning light

turns off (about one minute). After

Step 2, wait an additional 5 seconds

before proceeding with Step 3. Once

you start Step 3, you must complete

the procedure within 30 seconds.

3. For the seating position you are

switching off, buckle then unbuckle the

seatbelt three times at a moderate

speed, ending in the unbuckled state.

After Step 3, the seatbelt warning light

turns on.

37

Mustang (

4. While the seatbelt warning light is on,

buckle and then unbuckle the seatbelt.

After Step 4, the seatbelt warning light

flashes for confirmation.

• This will switch the feature off for that

seating position if it is currently on.

• This will switch the feature on for that

seating position if it is currently off.